Unit 6: Microcontroller Systems — Week 2 Assessment Form

From PC to Embedded Controller (BTEC Criteria A1, B1)

# Section 1: Student Information

1. Full Name:
__________________________________________________

2. Student ID / Group Cohort:
__________________________________________________

# Section 2: Diagnostic Engineering Toolbox

3. What is the fundamental difference between a digital and an analogue signal?

A. Digital has discrete 2-state logic (0V/5V), while analogue varies continuously.

B. Analogue signals can only have 0V or 5V values.

C. Digital signals are only found in power plants.

4. Why does an LED require a current-limiting resistor (e.g. 220 Ohm)?

A. To prevent excessive current from damaging the LED and microcontroller pin.

B. To step up 5V to 12V.

C. To convert direct current into alternating current.

5. In the Input -> Processing -> Output control model, what role does a push button play?

A. An Input sensor providing binary voltage state (HIGH/LOW).

B. A Processing CPU unit executing instructions.

C. An Output actuator creating light or mechanical motion.

6. What is the maximum recommended continuous DC current output from an Arduino Uno I/O pin?

A. 20 mA (40 mA absolute max rating)

B. 500 mA

C. 2.5 A

# Section 3: PC vs Microcontroller Architecture

7. Compare the role of an Operating System (PC) with Bare-Metal Firmware (Arduino):

________________________________________________________________________________________________________________________

8. State the primary purpose of a microcontroller in industrial engineering:

A. Special-purpose computer designed to control physical hardware and real-time processes directly.

B. General-purpose computer built for multi-monitor video editing and high-volume data streaming.

C. A device used solely for storage backup.

# Section 4: Arduino Uno Rev3 Datasheet Scavenger Hunt

9. What is the total internal Flash program memory size on the ATmega328P?

A. 32 KB (0.5 KB used by bootloader)

B. 512 MB

C. 2 KB

10. What is the internal SRAM working memory size on the ATmega328P?

A. 2 KB

B. 32 KB

C. 16 MB

11. What is the recommended external DC input voltage range via the Barrel Jack / VIN?

A. 7 V to 12 V DC

B. 1.5 V to 3.3 V DC

C. 24 V to 48 V DC

12. What clock frequency is provided by the external crystal oscillator?

A. 16 MHz

B. 3.2 GHz

C. 100 kHz

# Section 5: Practical Programming & Circuit Design Evidence

13. Explain the execution difference between void setup() and void loop():

________________________________________________________________________________________________________________________

14. Why is INPUT_PULLUP used when connecting a pushbutton switch?

A. It activates the internal 20k-50k Ohm pull-up resistor to 5V, preventing floating input states.

B. It doubles the clock speed of the processor.

C. It converts the button into an analogue potentiometer.

15. Paste your verified C++ sketch for the Dual-Button & Dual-LED Start/Stop Station:

________________________________________________________________________________________________________________________
________________________________________________________________________________________________________________________
________________________________________________________________________________________________________________________

16. [File Upload] Upload screenshot of your Tinkercad Circuit Simulation (Evidence of schematic wiring and simulation run):

[ File Upload Item: Student attaches .png / .jpg / .pdf ]

17. [File Upload] Upload clear photograph of your Physical Breadboard & Arduino Build (Evidence of hardware assembly and operation):

[ File Upload Item: Student attaches .png / .jpg / .pdf ]